# 🌱 FARM BOT – TEXT BLOCK DIAGRAM (WITH PIN CONNECTIONS)

## 🔋 POWER DISTRIBUTION (CORE OF THE SYSTEM)

12V BATTERY
│
├──► L298N MOTOR DRIVER (12V input)
│
├──► BUCK CONVERTER #1 (LM2596) → 5V
│       ├──► ESP32 (VIN / 5V pin)
│       ├──► Soil Moisture Sensor (VCC)
│       └──► LCD (if used later)
│
└──► BUCK CONVERTER #2 (LM2596) → 5.5–6V
        └──► PCA9685 Servo Driver (V+)
                └──► 3 Servo Motors

⚠️ ALL GNDs MUST BE COMMON (Battery GND, ESP32 GND, L298N GND, PCA9685 GND)

## 🧠 MAIN CONTROLLER

### ESP32

ESP32
│
├── GPIO14 → L298N IN1
├── GPIO27 → L298N IN2
├── GPIO26 → L298N IN3
├── GPIO25 → L298N IN4
│
├── GPIO33 → L298N ENA (PWM – Left wheels)
├── GPIO32 → L298N ENB (PWM – Right wheels)
│
├── GPIO21 → PCA9685 SDA
├── GPIO22 → PCA9685 SCL
│
├── GPIO34 → Soil Moisture Sensor (Analog)
│
├── GPIO18 → Pump Motor Relay / MOSFET
└── GPIO19 → Cutter Motor Relay / MOSFET

## 🚗 WHEEL DRIVE SYSTEM

### L298N

L298N
│
├── IN1 ← ESP32 GPIO14
├── IN2 ← ESP32 GPIO27
├── IN3 ← ESP32 GPIO26
├── IN4 ← ESP32 GPIO25
│
├── ENA ← ESP32 GPIO33
├── ENB ← ESP32 GPIO32
│
├── OUT1 + OUT2 → Left Side DC Motors (200 RPM)
├── OUT3 + OUT4 → Right Side DC Motors (200 RPM)
│
├── +12V → Battery 12V
└── GND  → Common GND

### 🛞 Motor Mapping

Left Front  + Left Rear → OUT1 / OUT2
Right Front + Right Rear → OUT3 / OUT4

## 🦾 ARM CONTROL SYSTEM

### PCA9685

PCA9685
│
├── VCC → ESP32 3.3V
├── GND → Common GND
├── SDA → ESP32 GPIO21
├── SCL → ESP32 GPIO22
│
├── V+ → Buck Converter #2 (5.5–6V)
│
├── Channel 0 → Servo 1 (Base – Left / Right)
├── Channel 1 → Servo 2 (Arm – Up / Down)
└── Channel 2 → Servo 3 (Gripper – Open / Close)

## 🦾 SERVO MOTORS (TowerPro MG995)

Servo Wire Colors:
Brown / Black → GND
Red           → 5.5–6V (from PCA9685 V+)
Yellow        → Signal (PCA9685 channel)

## 💧 WATER PUMP SYSTEM

Pump Motor
│
├── + → External 12V (via Relay / MOSFET)
├── - → GND
│
Relay / MOSFET Control Pin
└── ESP32 GPIO18

📌 Do NOT connect pump directly to ESP32

## 🌾 CUTTER MOTOR SYSTEM

Cutter DC Motor
│
├── + → External 12V (via Relay / MOSFET)
├── - → GND
│
Relay / MOSFET Control Pin
└── ESP32 GPIO19

## 🌱 SOIL MOISTURE SENSOR

Soil Moisture Sensor
│
├── VCC → 5V (Buck Converter #1)
├── GND → Common GND
└── AO  → ESP32 GPIO34 (Analog Input)

📱 Data sent to mobile phone via WiFi web interface

## 📡 COMMUNICATION

Mobile Phone
   ⇅ WiFi
ESP32 (SoftAP / WebServer)

- Shows soil moisture value

- Controls wheels, arm, pump, cutter