Below is Output-1: PIN CONFIGURATION, written as a deployment-ready hardware reference.
This section is suitable for final wiring, lab assembly, documentation, and project reports.

1) PIN CONFIGURATION & POWER ARCHITECTURE

1.1 Controller Overview

- Main Controller: ESP32

- Servo Controller: PCA9685

- Motor Drivers: Dual H-Bridge DC Motor Drivers (L298N / equivalent)

1.2 ESP32 Pin Mapping (Complete)

| ESP32 GPIO | Connected Module | Signal Purpose | Notes |
| --- | --- | --- | --- |
| GPIO 14 | Motor Driver A | IN1 | Left Motor Forward |
| GPIO 27 | Motor Driver A | IN2 | Left Motor Backward |
| GPIO 26 | Motor Driver A | IN3 | Right Motor Forward |
| GPIO 25 | Motor Driver A | IN4 | Right Motor Backward |
| GPIO 33 | Motor Driver A | ENA (PWM) | Left motor speed |
| GPIO 32 | Motor Driver A | ENB (PWM) | Right motor speed |
| GPIO 18 | Motor Driver B | IN1 | Scrubber DC motor |
| GPIO 19 | Motor Driver B | IN2 | Scrubber DC motor |
| GPIO 21 | PCA9685 | SDA | I²C Data |
| GPIO 22 | PCA9685 | SCL | I²C Clock |
| GPIO 5 | Ultrasonic Sensor | TRIG | Trigger pulse |
| GPIO 17 | Ultrasonic Sensor | ECHO | Distance measurement |
| GPIO 23 | Buzzer | Alert signal | Obstacle alert |
| GND | All Modules | Common Ground | Mandatory |
| 5V | Sensors / PCA9685 | Logic Power | From buck converter |
| 3.3V | ESP32 internal | Logic | Do NOT power servos |

1.3 PCA9685 Servo Channel Allocation

| PCA9685 Channel | Actuator | Description |
| --- | --- | --- |
| Channel 0 | Base Rotation Servo | Left ↔ Right (0–180°) |
| Channel 1 | Arm Lift Servo | Up ↔ Down (0–180°) |
| Channel 2 | Gripper Servo | Grab / Release |
| Channel 3 | Water Wiper Servo | Up / Down |
| Channel 4 | Scrubber Lift Servo | Up / Down |
| Channel 5–15 | Reserved | Future expansion |

1.4 PCA9685 Electrical Connections

| PCA9685 Pin | Connected To | Purpose |
| --- | --- | --- |
| VCC | 5V Buck Output | Logic power |
| GND | Common Ground | Signal reference |
| SDA | ESP32 GPIO 21 | I²C Data |
| SCL | ESP32 GPIO 22 | I²C Clock |
| V+ | 6V Buck Output | Servo power rail |

Important:

- VCC ≠ V+

- VCC powers logic, V+ powers servos

1.5 Motor Driver Pin Mapping

Motor Driver A (Rover Wheels)

| Driver Pin | ESP32 Pin | Function |
| --- | --- | --- |
| IN1 | GPIO 14 | Left wheel forward |
| IN2 | GPIO 27 | Left wheel backward |
| IN3 | GPIO 26 | Right wheel forward |
| IN4 | GPIO 25 | Right wheel backward |
| ENA | GPIO 33 (PWM) | Left speed |
| ENB | GPIO 32 (PWM) | Right speed |
| Vmotor | 12V Battery | Motor power |
| GND | Common Ground | Reference |

Motor Driver B (Scrubber Motor)

| Driver Pin | ESP32 Pin | Function |
| --- | --- | --- |
| IN1 | GPIO 18 | Scrubber rotate |
| IN2 | GPIO 19 | Scrubber rotate |
| EN | Jumper / PWM optional | Speed control |
| Vmotor | 12V Battery | Motor power |
| GND | Common Ground | Reference |

1.6 Ultrasonic Sensor Pin Mapping

| Ultrasonic Pin | ESP32 Pin | Purpose |
| --- | --- | --- |
| VCC | 5V Buck Output | Sensor power |
| TRIG | GPIO 5 | Trigger signal |
| ECHO | GPIO 17 | Distance echo |
| GND | Common Ground | Reference |

Software Safety Note:
ECHO pin must be read using timeout logic to avoid blocking ESP32.

1.7 Buzzer Pin Mapping

| Buzzer Pin | ESP32 Pin | Function |
| --- | --- | --- |
| + | GPIO 23 | Alert signal |
| – | GND | Ground |

Buzzer activates automatically when obstacle detected.

1.8 Buck Converter Usage (CRITICAL)

Buck Converter #1 – Servo Power

- Input: 12V Battery

- Output: 6V (High current)

- Used For: All servo motors via PCA9685 V+

Buck Converter #2 – Logic Power

- Input: 12V Battery

- Output: 5V

- Used For: ESP32, PCA9685 VCC, ultrasonic sensor, buzzer

1.9 Power Distribution Architecture

| Voltage Level | Source | Used By |
| --- | --- | --- |
| 12V | Battery | DC motors, buck converters |
| 6V | Buck Converter #1 | All servo motors |
| 5V | Buck Converter #2 | ESP32 VIN, PCA9685 VCC, sensors |
| 3.3V | ESP32 internal | ESP32 logic only |

Mandatory Power Rules

- Never power servos from ESP32

- All grounds must be common

- Use thick wires for servo V+ and DC motors

- Keep logic wiring separate from motor wiring

1.10 Safety & Stability Notes

- Servo jitter avoided using PCA9685

- ESP32 protected from voltage dips

- Obstacle detection has highest priority interrupt

- Emergency stop overrides all outputs