Below is a clear, simple, competition- and user-ready USER MANUAL, written so that non-technical users, caretakers, and judges can easily understand and operate your robot.

Language is simple, step-by-step, and suitable for handicapped users and their assistants.

5) USER MANUAL

Rovering Robotic Arm for Assistive Applications

1. INTRODUCTION

This robot is designed to help physically handicapped users perform daily tasks such as:

- Moving objects

- Reaching items

- Cleaning surfaces

- Scrubbing utensils

The robot is controlled wirelessly using a mobile phone and does not require any mobile application.

The system is powered and controlled using ESP32 with a simple web interface.

2. WHAT YOU NEED TO OPERATE THE ROBOT

Before starting, make sure you have:

- Fully charged 12V battery

- A smartphone with Wi-Fi and a browser (Chrome recommended)

- Flat surface for safe movement

3. POWERING THE ROBOT (STEP-BY-STEP)

- Place the robot on a flat surface

- Ensure:

  - All motors are connected properly

  - No wires are loose

- Turn ON the main power switch

- You will notice:

  - ESP32 power LED turns ON

  - Servo driver LED turns ON

  - Robot is now ready

Important:
Do not touch moving parts while powering ON.

4. CONNECTING TO THE ROBOT (ESP32 INTERFACE)

Step 1: Connect Wi-Fi

- Open Wi-Fi settings on your phone

- Look for network name:
AssistBot

- Connect using password:
12345678

Step 2: Open Control Page

- Open your mobile browser

- Type the address:
192.168.4.1

- The robot control screen will appear

No app installation is required.

5. DRIVING THE ROVER

Buttons Available

- Forward

- Backward

- Left

- Right

- STOP

How to Use

- Touch and hold a direction button → Robot moves

- Release the button → Robot stops

- Press STOP for emergency stop

Safety Feature

- If an obstacle is detected:

  - Robot stops automatically

  - Buzzer sound is activated

6. USING THE ROBOTIC ARM

Arm Controls

- Rotate Left

- Rotate Right

- Arm Up

- Arm Down

How to Use

- Touch and hold to move slowly

- Release to stop immediately

- Movements are smooth and controlled for safety

7. USING THE GRIPPER

Buttons

- Grab

- Release

How to Use

- Position the arm near the object

- Press Grab to hold the object

- Press Release to drop the object gently

8. USING THE WATER WIPER

Buttons

- Wiper Up

- Wiper Down

How to Use

- Move the wiper down to the surface

- Move it up when finished

- Useful for wiping tables, plates, or surfaces

9. USING THE SCRUBBER SYSTEM

Scrubber Lift (Servo)

- Scrubber Up

- Scrubber Down

Scrubber Rotation (Motor)

- Scrubber ON

- Scrubber OFF

Correct Usage Order

- Move scrubber Down

- Press Scrubber ON

- Perform cleaning

- Press Scrubber OFF

- Move scrubber Up

Note:
Do not move the rover while scrubbing.

10. SAFETY PRECAUTIONS (VERY IMPORTANT)

- Keep hands away from moving parts

- Do not overload the gripper

- Avoid water contact with electronics

- Always place robot on stable surface

- Stop robot immediately if abnormal behavior occurs

- Do not operate with loose wires

- Do not exceed recommended voltage levels

11. TROUBLESHOOTING GUIDE

Problem: Robot does not move

- Check battery charge

- Check motor connections

- Ensure Wi-Fi is connected

Problem: Servos not moving

- Check servo power (6V supply)

- Ensure PCA9685 is powered

- Check common ground connection

Problem: Web page not opening

- Confirm phone is connected to AssistBot Wi-Fi

- Re-enter 192.168.4.1

- Restart the robot

Problem: Buzzer always ON

- Check ultrasonic sensor alignment

- Ensure no object is too close

12. MAINTENANCE TIPS

- Recharge battery regularly

- Clean scrubber after use

- Tighten loose screws monthly

- Keep electronics dry

- Inspect wires for damage

- Store robot in dry, dust-free place

13. USER CARE RECOMMENDATIONS

- Use with supervision if needed

- Train user with simple tasks first

- Adjust speed and movement gradually

- Encourage independence safely

14. CONCLUSION

This robot is designed to improve independence, reduce physical strain, and support daily activities for physically handicapped users.

With proper use and care, it can become a reliable assistive companion.