/************************************************************

* ASSISTIVE ROVERING ROBOTIC ARM

* ESP32 + PCA9685 + MOBILE WEB UI

* Includes SCRUBBER LIFT SERVO (UP/DOWN)

************************************************************/

#include <WiFi.h>

#include <WebServer.h>

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

/******************** WIFI ********************/

const char* ssid = "AssistBot";

const char* password = "12345678";

WebServer server(80);

/******************** PCA9685 ********************/

Adafruit_PWMServoDriver pca = Adafruit_PWMServoDriver(0x40);

#define SERVOMIN 120

#define SERVOMAX 520

#define SERVO_STEP 2

#define SERVO_DELAY 15

/* Servo channels */

#define BASE_SERVO   0

#define ARM_SERVO    1

#define GRIP_SERVO   2

#define WIPER_SERVO  3

#define SCRUB_SERVO  4   // NEW: Scrubber up/down servo

/******************** MOTOR PINS ********************/

#define IN1 14

#define IN2 27

#define IN3 26

#define IN4 25

#define ENA 33

#define ENB 32

#define SCRUB_IN1 18

#define SCRUB_IN2 19

/******************** ULTRASONIC & BUZZER ********************/

#define TRIG_PIN 5

#define ECHO_PIN 17

#define BUZZER   23

/******************** SERVO STATES ********************/

int baseAngle  = 90;

int armAngle   = 90;

int gripAngle  = 90;

int wiperAngle = 90;

int scrubAngle = 90;   // NEW

bool obstacleDetected = false;

/******************** HTML UI ********************/

const char webpage[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>Assistive Robot</title>

<style>

body { font-family: Arial; background:#111; color:#fff; text-align:center; }

h2 { margin:10px 0; }

button {

width:45%; height:70px; font-size:18px;

margin:5px; border:none; border-radius:10px;

background:#007BFF; color:white;

}

button:active { background:#0056b3; }

.stop { background:#ff3333; width:92%; }

</style>

<script>

let interval;

function send(cmd){

fetch(`/cmd?action=${cmd}`);

}

function hold(cmd){

send(cmd);

interval=setInterval(()=>send(cmd),200);

}

function release(){

clearInterval(interval);

send('STOP');

}

</script>

</head>

<body>

<h2>ROVER CONTROL</h2>

<button onmousedown="hold('F')" onmouseup="release()" ontouchstart="hold('F')" ontouchend="release()">Forward</button><br>

<button onmousedown="hold('L')" onmouseup="release()" ontouchstart="hold('L')" ontouchend="release()">Left</button>

<button onmousedown="hold('R')" onmouseup="release()" ontouchstart="hold('R')" ontouchend="release()">Right</button><br>

<button onmousedown="hold('B')" onmouseup="release()" ontouchstart="hold('B')" ontouchend="release()">Backward</button>

<button class="stop" onclick="send('STOP')">STOP</button>

<h2>ARM CONTROL</h2>

<button onmousedown="hold('BASE_L')" onmouseup="release()" ontouchstart="hold('BASE_L')" ontouchend="release()">Rotate Left</button>

<button onmousedown="hold('BASE_R')" onmouseup="release()" ontouchstart="hold('BASE_R')" ontouchend="release()">Rotate Right</button><br>

<button onmousedown="hold('ARM_U')" onmouseup="release()" ontouchstart="hold('ARM_U')" ontouchend="release()">Arm Up</button>

<button onmousedown="hold('ARM_D')" onmouseup="release()" ontouchstart="hold('ARM_D')" ontouchend="release()">Arm Down</button>

<h2>GRIPPER</h2>

<button onclick="send('GRAB')">Grab</button>

<button onclick="send('RELEASE')">Release</button>

<h2>CLEANING</h2>

<button onmousedown="hold('WIPER_U')" onmouseup="release()" ontouchstart="hold('WIPER_U')" ontouchend="release()">Wiper Up</button>

<button onmousedown="hold('WIPER_D')" onmouseup="release()" ontouchstart="hold('WIPER_D')" ontouchend="release()">Wiper Down</button><br>

<button onmousedown="hold('SCRUB_U')" onmouseup="release()" ontouchstart="hold('SCRUB_U')" ontouchend="release()">Scrubber Up</button>

<button onmousedown="hold('SCRUB_D')" onmouseup="release()" ontouchstart="hold('SCRUB_D')" ontouchend="release()">Scrubber Down</button><br>

<button onclick="send('SCRUB_ON')">Scrubber ON</button>

<button onclick="send('SCRUB_OFF')">Scrubber OFF</button>

</body>

</html>

)rawliteral";

/******************** FUNCTIONS ********************/

void moveServo(uint8_t ch, int &cur, int target){

target = constrain(target, 0, 180);

int step = (target > cur) ? SERVO_STEP : -SERVO_STEP;

while(cur != target){

cur += step;

if(abs(target - cur) < SERVO_STEP) cur = target;

pca.setPWM(ch,0,map(cur,0,180,SERVOMIN,SERVOMAX));

delay(SERVO_DELAY);

}

}

void stopRover(){

digitalWrite(IN1,LOW); digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW); digitalWrite(IN4,LOW);

}

void checkObstacle(){

digitalWrite(TRIG_PIN,LOW); delayMicroseconds(2);

digitalWrite(TRIG_PIN,HIGH); delayMicroseconds(10);

digitalWrite(TRIG_PIN,LOW);

long d=pulseIn(ECHO_PIN,HIGH,25000);

int dist=d*0.034/2;

if(dist>0 && dist<25){

obstacleDetected=true;

stopRover();

digitalWrite(BUZZER,HIGH);

} else {

obstacleDetected=false;

digitalWrite(BUZZER,LOW);

}

}

/******************** SETUP ********************/

void setup(){

pinMode(IN1,OUTPUT); pinMode(IN2,OUTPUT);

pinMode(IN3,OUTPUT); pinMode(IN4,OUTPUT);

pinMode(ENA,OUTPUT); pinMode(ENB,OUTPUT);

pinMode(SCRUB_IN1,OUTPUT); pinMode(SCRUB_IN2,OUTPUT);

pinMode(TRIG_PIN,OUTPUT); pinMode(ECHO_PIN,INPUT);

pinMode(BUZZER,OUTPUT);

Wire.begin(21,22);

pca.begin();

pca.setPWMFreq(50);

WiFi.softAP(ssid,password);

server.on("/",[](){ server.send_P(200,"text/html",webpage); });

server.on("/cmd",[](){

String a=server.arg("action");

/* Rover */

if(a=="F" && !obstacleDetected){digitalWrite(IN1,HIGH);digitalWrite(IN3,HIGH);}

else if(a=="B"){digitalWrite(IN2,HIGH);digitalWrite(IN4,HIGH);}

else if(a=="L"){digitalWrite(IN2,HIGH);digitalWrite(IN3,HIGH);}

else if(a=="R"){digitalWrite(IN1,HIGH);digitalWrite(IN4,HIGH);}

else if(a=="STOP"){stopRover();}

/* Arm */

else if(a=="BASE_L") moveServo(BASE_SERVO,baseAngle,baseAngle-10);

else if(a=="BASE_R") moveServo(BASE_SERVO,baseAngle,baseAngle+10);

else if(a=="ARM_U")  moveServo(ARM_SERVO,armAngle,armAngle+10);

else if(a=="ARM_D")  moveServo(ARM_SERVO,armAngle,armAngle-10);

/* Gripper */

else if(a=="GRAB")    moveServo(GRIP_SERVO,gripAngle,40);

else if(a=="RELEASE")moveServo(GRIP_SERVO,gripAngle,90);

/* Wiper */

else if(a=="WIPER_U") moveServo(WIPER_SERVO,wiperAngle,wiperAngle+10);

else if(a=="WIPER_D") moveServo(WIPER_SERVO,wiperAngle,wiperAngle-10);

/* Scrubber lift (NEW) */

else if(a=="SCRUB_U") moveServo(SCRUB_SERVO,scrubAngle,scrubAngle+10);

else if(a=="SCRUB_D") moveServo(SCRUB_SERVO,scrubAngle,scrubAngle-10);

/* Scrubber motor */

else if(a=="SCRUB_ON"){digitalWrite(SCRUB_IN1,HIGH);digitalWrite(SCRUB_IN2,LOW);}

else if(a=="SCRUB_OFF"){digitalWrite(SCRUB_IN1,LOW);digitalWrite(SCRUB_IN2,LOW);}

server.send(200,"text/plain","OK");

});

server.begin();

}

/******************** LOOP ********************/

void loop(){

server.handleClient();

checkObstacle();

}

✅ WHAT YOU HAVE NOW

Rover movement

3-DOF robotic arm

Gripper

Water wiper

Scrubber rotation motor

Scrubber lift servo (up/down)

Obstacle safety

Buzzer alert

Mobile-friendly UI

Single ESP32 program

This is now a complete assistive cleaning robot.